18001

PRACTICAL NO : 01

Study of Networking Device

Q1. Identify the network address & subnet mask for the ip address 192.168.1.0

Given ip address is 192.168.1.0

The subnet mask is 255.255.255.0 as it is belong to class C

In Binary Form:-

11111111 .11111111 .11111111.00000000 In this the 1st 24 bits represent the host part.

Q2. Create 4 Subnets:-

To create 4 subnets we need to binary two bits from the host part.

2^2=4

So the new subnet mask is 255.255.255.192

Binary Form:-

11111111.11111111.11111111.11000000

so 26 bits for the network part 6 bits for the host part. Total no. of subnet 4.

[ combining 2 or more subnets will give us the Supernet.]

Total no. Of useable ip address in each subnet is 64- -2 because 1 address is for network

and 1 for broadcast, so subtracting them. 62 IP addresses are usable.

CIDR format:-classless InterDomain Representation.

New Subnet Mask:- 255.255.255.192/126

The 1st subnet mask is 192.168.1.0/126

The 2nd subnet mask is 192.168.1.64/126

Subnet 1=192.168.1.0 ---> Network address

Useable host address 192.168.1.62/126

Broadcast address 192.168.1.63/26

Subnet 2 192.168.1.64

1st host address 192.168.1.65

Last host address 192.168.1.126/26

Broadcast address 192.168.1.127

Subnet 3 192.168.1.128

Last address is 192.168.1.190

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 01

18001

15th address is 192.168.1.129

Subnet 4 192.168.1.192

Last address is 192.168.1.254

Broadcast address 192.168.1.255

Q3. A network address 192.168.1.0/24 creates 3 subnets with subnet 1 corrosting if so

subnet 2 with 20, and subnet 3 with 10.

(Since the no. The host bit is determined by the ip address. So ip address formula to

calculate no. Of bits required to satisfy each subnet ip address is 2^n/ no. of ip address

required.

Where n is the no. of bits and +2 represent n|6)

Subnet 1 of 50 ip address

Ex:- 2^6 = 64 with 62 usable ip addresses we need 6 host bits.

Subnet mask is 255.255.255.192/26

2^5 = 32 ,30 useable

Subnet mask is 255.255.255.224|27

2^4=16

Subnet 255.255.255.140|28

Subnet 1

Subnet mask: 255.255.255.192|26

Network address 192.168.1.0|26

Host address 192.168.1.1|26

Last host address 192.168.1.50|26

Useable ip address 62//

Broadcast 192.168.1.63|26

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 01

18001

PRACTICAL NO: 02

Demonstrate the Static Routing in Packet Tracer.

AIM :- To configure and demonstrate static routing between two networks using two routers in Packet Tracer.

STEPS :-

- In  packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via FastEthernet0/0 using cross-copper cord and Router1 with PC1 via FastEthernet0/0 using cross-copper cord.

- Connect both routers via FastEthernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in IP configuration of both PCs respectively.

- TOPOLOGY :-

- Code

- Router0 configuration

- Router>enable

- Router#configure terminal

- Router(config)#interface FastEthernet0/0

- Router(config-if)#ip address 10.0.0.1 255.0.0.0

- Router(config-if)#no shutdown

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

- Router(config-if)#exit

- Router(config)#interface FastEthernet0/1

- Router(config-if)#ip address 20.0.0.1 255.0.0.0

- Router(config-if)#no shutdown

- Router(config-if)#exit

- Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2

- Router(config)#exit

- Router1 configuration

- Router>enable

- Router#configure terminal

- Router(config)#interface FastEthernet0/1

- Router(config-if)#ip address 20.0.0.2 255.0.0.0

- Router(config-if)#no shutdown

- Router(config-if)#exit

- Router(config)#interface FastEthernet0/0

- Router(config-if)#ip address 40.0.0.1 255.0.0.0

- Router(config-if)#no shutdown

- Router(config-if)#exit

- Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1

- Router(config)#exit

- PC 0 :

- C:\>ipconfig

- C:\>ping 10.0.0.1

- C:\>ping 20.0.0.1

- C:\>ping 20.0.0.2

- C:\>ping 40.0.0.1

- C:\>ping 40.0.0.2

- PC 1:

- C:\>ipconfig

- C:\>ping 40.0.0.1

- C:\>ping 20.0.0.2

- C:\>ping 20.0.0.1

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

- C:\>ping 10.0.0.1

- C:\>ping 10.0.0.2

- OUTPUT:-

- PC 0:

- PC 1:

- Conclusion :

- The above network topology has been executed successfully and static routing has been demonstrated. Communication between PC0 (10.0.0.2) and PC1 (40.0.0.2) across two routers has been verified with successful ping results.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

- Q2.Demonstrate the OSPF Routing in Packet Tracer.

AIM :- To configure and demonstrate OSPF dynamic routing protocol between two networks using two routers in Packet Tracer.

STEPS :-

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via FastEthernet0/0 using cross-copper cord and Router1 with PC1 via FastEthernet0/0 using cross-copper cord.

- Connect both routers via FastEthernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in IP configuration of both PCs respectively.

- Configure OSPF routing protocol on both routers.

TOPOLOGY:-

Code :

Router0 configuration :

Router>enable

Router#configure terminal

Router(config)#hostname Router0

Router0(config)#interface FastEthernet0/0

Router0(config-if)#ip address 192.168.1.1 255.255.255.0

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface FastEthernet0/1

Router0(config-if)#ip address 10.0.0.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#router ospf 1

Router0(config-router)#network 192.168.1.0 0.0.0.255 area 0

Router0(config-router)#network 10.0.0.0 0.0.0.255 area 0

Router0(config-router)#exit

Router0(config)#exit

Router0#copy running-config startup-config

Router1 configuration :

Router>enable

Router#configure terminal

Router(config)#hostname Router1

Router1(config)#interface FastEthernet0/1

Router1(config-if)#ip address 10.0.0.2 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface FastEthernet0/0

Router1(config-if)#ip address 172.16.1.1 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#router ospf 1

Router1(config-router)#network 10.0.0.0 0.0.0.255 area 0

Router1(config-router)#network 172.16.1.0 0.0.0.255 area 0

Router1(config-router)#exit

Router1(config)#exit

Router1#copy running-config startup-config

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

PC0 configuration :

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PC1 Configuration :

- IP Address: 172.16.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 172.16.1.1

PC0 Command Prompt :

C:\>ipconfig

C:\>ping 192.168.1.1

C:\>ping 10.0.0.1

C:\>ping 10.0.0.2

C:\>ping 172.16.1.1

C:\>ping 172.16.1.2

PC1 Command Prompt :

C:\>ipconfig

C:\>ping 172.16.1.1

C:\>ping 10.0.0.2

C:\>ping 10.0.0.1

C:\>ping 192.168.1.1

C:\>ping 192.168.1.2

Router Verification Command

Router0#show ip route

Router0#show ip ospf neighbour

Router0#show ip ospf database

Router1#show ip route

Router1#show ip ospf neighbour

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

OUTPUT:

PC0 OUTPUT :

PC1 OUTPUT :

OSPF Neighbour Output:

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

IP Route Output:

Conclusion : The above network topology has been executed successfully and OSPF dynamic routing has been demonstrated.

Q3. Demonstrate the RIP Routing in Packet Tracer.

AIM :- To configure and demonstrate RIP dynamic routing protocol between two networks using two routers in Packet Tracer.

STEPS :-

- In packet tracer place 2 PCs and 2 Routers.

- Connect Router0 with PC0 via FastEthernet0/0 using cross-copper cord and Router1 with PC1 via FastEthernet0/0 using cross-copper cord.

- Connect both routers via FastEthernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in IP configuration of both PCs respectively.

- Configure RIP version 2 routing protocol on both routers.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

TOPOLOGY :-

Code:

Router0 Configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router0

Router0(config)#interface FastEthernet0/0

Router0(config-if)#ip address 192.168.10.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface FastEthernet0/1

Router0(config-if)#ip address 10.10.10.1 255.255.255.0

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#router rip

Router0(config-router)#version 2

Router0(config-router)#network 192.168.10.0

Router0(config-router)#network 10.10.10.0

Router0(config-router)#no auto-summary

Router0(config-router)#exit

Router0(config)#exit

Router0#copy running-config startup-config

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

Router1 configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router1

Router1(config)#interface FastEthernet0/1

Router1(config-if)#ip address 10.10.10.2 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface FastEthernet0/0

Router1(config-if)#ip address 172.16.20.1 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#router rip

Router1(config-router)#version 2

Router1(config-router)#network 10.10.10.0

Router1(config-router)#network 172.16.20.0

Router1(config-router)#no auto-summary

Router1(config-router)#exit

Router1(config)#exit

Router1#copy running-config startup-config

PC0 Configuration:

- IP Address: 192.168.10.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

PC1 Configuration:

- IP Address: 172.16.20.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 172.16.20.1

PCO Command Prompt :

C:\>ipconfig

C:\>ping 192.168.10.1

C:\>ping 10.10.10.1

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

C:\>ping 10.10.10.2

C:\>ping 172.16.20.1

C:\>ping 172.16.20.2

PC0 Command Prompt :

C:\>ipconfig

C:\>ping 172.16.20.1

C:\>ping 10.10.10.2

C:\>ping 10.10.10.1

C:\>ping 192.168.10.1

C:\>ping 192.168.10.2

Router Verification Commands:

Router0#show ip route

Router0#show ip protocols

Router0#show ip rip database

Router1#show ip route

Router1#show ip protocols

OUTPUT :

PC0 OUTPUT :

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

PC1 OUTPUT :

IP ROUTE OUTPUT :

RIP PROTOCOL OUTPUT :

CONCLUSION : The above network topology has been executed successfully and RIP version 2 dynamic routing has been demonstrated.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 02

18001

PRACTICAL NO 03

Q1. Demonstrate DHCP Server in Cisco Packet Tracer.

AIM :- To configure a router as DHCP server to automatically assign IP addresses to clients.

Devices Required:

- 1 Router (1841 or any model)

- 1 Switch (2960 or any model)

- 2-4 PCs

Connections:

- Router FastEthernet0/0 to Switch FastEthernet0/1 (Straight-through cable)

- PC0 to Switch FastEthernet0/2 (Straight-through cable)

- PC1 to Switch FastEthernet0/3 (Straight-through cable)

Topology :

Code :

Router Configuration Commands:

Router>enable

Router#configure terminal

Router(config)#hostname DHCP-Router

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

DHCP-Router(config)#interface FastEthernet0/0

DHCP-Router(config-if)#ip address 10.0.0.1 255.255.255.0

DHCP-Router(config-if)#no shutdown

DHCP-Router(config-if)#exit

DHCP Pool Configuration:

DHCP-Router(config)#ip dhcp pool STUDENT_NETWORK

DHCP-Router(dhcp-config)#network 10.0.0.0 255.255.255.0

DHCP-Router(dhcp-config)#default-router 10.0.0.1

DHCP-Router(dhcp-config)#dns-server 8.8.8.8

DHCP-Router(dhcp-config)#exit

Exclude IP Addresses:

DHCP-Router(config)#ip dhcp excluded-address 10.0.0.1 10.0.0.10

DHCP-Router(config)#exit

DHCP-Router#copy running-config startup-config

PC Configuration:

- On each PC, go to Desktop tab → IP Configuration

- Select DHCP option

Verification cammand :

On Router:

DHCP-Router#show ip dhcp pool

DHCP-Router#show ip dhcp binding

DHCP-Router#show ip interface brief

On  both PCs (Command Prompt):

C:\>ipconfig

C:\>ipconfig /all

C:\>ping 10.0.0.1

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Output Results:

PC0 - ipconfig:

PC1 - ipconfig:

Router - DHCP Bindings (WORKING COMMAND):

Router - DHCP Pool Status (WORKING COMMAND):

`

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Router - Interface Status:

Connectivity Test - PC0 to Router:

Conclusion: The DHCP server has been successfully configured on the router using commands compatible with Cisco Packet Tracer.

- Q2.Demonstrate DNS Server in Packet Tracer.

Aim:- To configure a DNS server to resolve domain names to IP addresses and test name resolution in the network.

Devices Needed:

- 1 Server (DNS Server)

- 1 Router

- 1 Switch

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

- 2 PCs (Clients)

- TOPOLOGY:-

- Code:

- Configure DNS Records:

- Name: www.company.com

- Type: A Record

- Address: 192.168.1.5    # Points to Server itself

- -> Add

- Name: ftp.company.com

- Type: A Record

- Address: 192.168.1.5    # Points to Server itself

- -> Add

- Name: mail.company.com

- Type: A Record

- Address: 192.168.1.5    # Points to Server itself

- -> Add

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001 Server IP Configuration:

- IP Address: 192.168.1.5

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- DNS Server: 192.168.1.5

- Router Configuration:

- Router>enable

- Router#configure terminal

- Router(config)#interface FastEthernet0/0

- Router(config-if)#ip address 192.168.1.1 255.255.255.0

- Router(config-if)#no shutdown

- Router(config-if)#exit

- Router(config)#exit

- Router#copy running-config startup-config

- PC0 Configuration:

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- DNS Server: 192.168.1.5

- PC1 Configuration:

- IP Address: 192.168.1.11

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- DNS Server: 192.168.1.5

- In PC Command Prompt

- PC0:

- C:\>nslookup www.company.com

- C:\>nslookup ftp.company.com

- C:\>nslookup mail.company.com

- C:\>ping www.company.com

- C:\>ping ftp.company.com

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

- PC1:

- C:\>nslookup www.company.com

- C:\>nslookup mail.company.com

- C:\>ping www.company.com

- C:\>ping mail.company.com

- Output:

- PC0 nslookup Results:

- C:\>nslookup www.company.com

- C:\>nslookup ftp.company.com

- C:\>nslookup mail.company.com

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

- PC0 Ping Results:

- C:\>ping www.company.com

- PC1 nslookup Results:

- C:\>nslookup mail.company.com

- C:\>nslookup www.company.com

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

- PC2 Ping Results:

- C:\>ping mail.company.com

- Conclusion: The DNS server has been configured successfully.

Q3. Demonstrate FTP Server in Packet Tracer.

Aim: To configure an FTP server and test file transfer between server and clients in the network.

Devices Needed:

- 1 Server (FTP Server)

- 1 Switch

- 2 PCs (FTP Clients)

Connection :

- Connect all devices to the switch using copper straight-through cables

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Topology:

Code:

Server Configuration (FTP):

- Click on Server → Services tab → FTP

- FTP Server Settings:

- FTP Service: ON

- Add User Account:

- Username: admin

- Password: cisco

- Check all permissions: Read/Write/List/Delete/Rename

- Click Add

Server IP Configuration:

- IP Address: 192.168.1.5

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

PC1 Configuration:

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PC2 Configuration:

- IP Address: 192.168.1.11

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

Create Test File on PC1

- Click on PC1 → Desktop tab → Text Editor

- Type: This is a test file for FTP transfer

- Save as: upload_test.txt

Verification: In PC Command Prompt

PC1 FTP Commands:

C:\>ftp 192.168.1.5

Username: admin

Password: cisco

ftp> dir

ftp> put upload_test.txt

ftp> dir

ftp> get welcome.txt

ftp> delete notes.txt

ftp> quit

PC2 FTP Commands:

C:\>ftp 192.168.1.5

Username: admin

Password: cisco

ftp> dir

ftp> get upload_test.txt

ftp> rename upload_test.txt downloaded_file.txt

ftp> dir

ftp> quit

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Ping Test (Both PCs):

C:\>ping 192.168.1.5

Output:

PC1 FTP Session:

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Ping Test Results:

PC2 FTP Session:

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Ping Test Results:

Conclusion: The FTP server has been configured successfully.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Q4. Demonstrate HTTP Server in Packet Tracer.

Aim: To configure a web server and test web page access from clients using web browsers.

Steps:

- In packet tracer place 1 Server-PT and 2 PC-PT and 1 Switch.

- Connect Server-PT with Switch via FastEthernet0 using straight-through copper cord.

- Connect PC-PT0 and PC-PT1 with Switch via FastEthernet0 using straight-through copper cords.

- Set the IP address of Server-PT and both PCs in IP configuration.

- Configure HTTP service on Server-PT and create web pages.

- Test web access from both PCs.

Topology:

Code:

Server-PT Configuration:

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

HTTP Service Configuration:

- HTTP Service: ON

- Created web pages: index.html, about.html, courses.html

Web Page Code:

index.html:

html

<!DOCTYPE html>

<html>

<head>

<title>My College Website</title>

</head>

<body>

<h1>Welcome to College Server</h1>

<p>This website is hosted on Server-PT in Packet Tracer.</p>

<a href="about.html">About College</a> |

<a href="courses.html">Courses</a>

</body>

</html>

about.html:

html

<!DOCTYPE html>

<html>

<head>

<title>About Our College</title>

</head>

<body>

<h1>About Our Institution</h1>

<p>Established in 1990, we provide quality technical education.</p>

<p><b>Departments:</b> Computer, IT, Electronics, Mechanical</p>

<a href="index.html">Home</a> |

<a href="courses.html">Our Courses</a>

</body>

</html>

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

courses.html:

html

<!DOCTYPE html>

<html>

<head>

<title>College Courses</title>

</head>

<body>

<h1>Available Courses</h1>

<ul>

<li>Computer Engineering</li>

<li>Information Technology</li>

<li>Electronics & Telecommunication</li>

<li>Mechanical Engineering</li>

</ul>

<a href="index.html">Home</a> |

<a href="about.html">About College</a>

</body>

</html>

PC-PT0 Configuration:

- IP Address: 192.168.1.20

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PC-PT1 Configuration:

- IP Address: 192.168.1.30

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

Verification: In Both PCs Web Browser

PC-PT0:

- Open Web Browser

- Enter URL: http://192.168.1.10

- Enter URL: http://192.168.1.10/about.html

- Enter URL: http://192.168.1.10/courses.html

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

PC-PT1:

- Open Web Browser

- Enter URL: http://192.168.1.10

- Enter URL: http://192.168.1.10/about.html

- Enter URL: http://192.168.1.10/courses.html

Connectivity Test:

C:\>ping 192.168.1.10

Output:

Ping Test Results:

Pc0 :

Pc1 :

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Web Browser Output:

Homepage (http://192.168.1.10) :

Pc0 :

Pc1 :

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

About Page (http://192.168.1.10/about.html):

Pc0 :

Pc1 :

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Courses Page (http://192.168.1.10/courses.html):

Pc0 :

Pc1 :

Conclusion: The HTTP web server has been configured successfully on Server-PT.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Q5. Demonstrate Telnet in Packet Tracer.

Aim: To configure Telnet on routers to enable remote administration and test remote connectivity between devices.

Devices Needed:

- 2 Routers

- 1 Switch

- 2 PCs (Telnet Clients)

Connection Guide:

- Connect Router0 and Router1 via serial cable (Serial DCE)

- Connect Router0 to Switch via FastEthernet

- Connect PCs to Switch via copper straight-through cables

Topology:

Code:

Router1 Configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router0

Router0(config)#interface FastEthernet0/0

Router0(config-if)#ip address 192.168.1.1 255.255.255.0

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#interface Serial0/0/0

Router0(config-if)#ip address 10.0.0.1 255.255.255.252

Router0(config-if)#clock rate 64000

Router0(config-if)#no shutdown

Router0(config-if)#exit

Router0(config)#line vty 0 4

Router0(config-line)#password cisco

Router0(config-line)#login

Router0(config-line)#exit

Router0(config)#enable secret class

Router0(config)#ip route 0.0.0.0 0.0.0.0 10.0.0.2

Router0(config)#exit

Router0#copy running-config startup-config

Router2 Configuration:

Router>enable

Router#configure terminal

Router(config)#hostname Router1

Router1(config)#interface Serial0/0/0

Router1(config-if)#ip address 10.0.0.2 255.255.255.252

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#interface FastEthernet0/0

Router1(config-if)#ip address 172.16.1.1 255.255.255.0

Router1(config-if)#no shutdown

Router1(config-if)#exit

Router1(config)#line vty 0 4

Router1(config-line)#password cisco

Router1(config-line)#login

Router1(config-line)#exit

Router1(config)#enable secret class

Router1(config)#ip route 192.168.1.0 255.255.255.0 10.0.0.1

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Router1(config)#exit

Router1#copy running-config startup-config

PC1 Configuration:

- IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

PC2 Configuration:

- IP Address: 192.168.1.11

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

Verification: In PC Command Prompt

PC1 Telnet Tests:

C:\>ping 192.168.1.1

C:\>ping 10.0.0.2

C:\>ping 172.16.1.1

C:\>telnet 192.168.1.1

C:\>telnet 10.0.0.2

PC2 Telnet Tests:

C:\>ping 192.168.1.1

C:\>ping 10.0.0.2

C:\>ping 172.16.1.1

C:\>telnet 192.168.1.1

C:\>telnet 172.16.1.1

Router Verification Commands:

Router#show users

Router#show line vty 0

Router#show running-config | include vty

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

Output:

PC1 Telnet to Router1:

telnet 192.168.1.1

PC1 Telnet to Router2:

C:\>telnet 10.0.0.2

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

PC2 Telnet to Router1:

C:\>telnet 192.168.1.1

Ping Test Results:

Router Show Users Output:

Conclusion: Telnet has been configured successfully on both routers.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 03

18001

PRACTICAL NO 04

STUDY OF CRYPTOGRAPHY SUBSTITUTION TECHNIQUES

AIM :- To study and Implement Substitution Techniques in a Cryptography using python to perform Encryption & Description of  a message.

PRORAM :-

def encrypt(text, shift):

result = ""

for char in text:

if char.isalpha():

base = 'A' if char.isupper() else 'a'

result += chr((ord(char) - ord(base) + shift) % 26 + ord(base))

else:

result += char

return result

def decrypt(text, shift):

return encrypt(text, -shift)

text = input("Enter plain text: ")

shift = int(input("Enter shift value: "))

encrypted_text = encrypt(text, shift)

print("Encrypted Text:", encrypted_text)

decrypted_text = decrypt(encrypted_text, shift)

print("Decrypted Text:", decrypted_text)

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 04

18001

OUTPUT :-

CONCLUSION  :-

The substitution technique replaces each letter in the plaintext with another letter based on a fixed shift value.

Through this practical, we learned how Caesar Cipher works and how basic encryption and decryption are implemented in Python.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 04

18001

PRACTICAL NO 05

STUDY OF SYMMETRIC KEY - DES & AES

AIM :- To Study and Implement Symmetric Key Cryptography using DES & AES Algorithm in Python.

ALORITHM OF (DES) :-

1. Import DES module.

2. Create a secret key and pad the message to match the block size.

3. Encrypt the plain text.

4. Decrypt the cipher-text.

5. Display the results.

PROGRAM :-

!pip install pycryptodome

from Crypto.Cipher import DES

from Crypto.Util.Padding import pad, unpad

key = b'12345678'

cipher = DES.new(key, DES.MODE_ECB)

plaintext = input("Enter message: ")

padded_text = pad(plaintext.encode(), DES.block_size)

encrypted_text = cipher.encrypt(padded_text)

print("Encrypted Text:", encrypted_text)

decrypted_text = unpad(cipher.decrypt(encrypted_text), DES.block_size)

print("Decrypted Text:", decrypted_text.decode())

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 05

18001

OUTPUT :-

ALORITHM OF (AES) :-

1. Import AES module.

2. Create a 16-byte secret key and pad the message.

3. Encrypt and then decrypt using AES.

4. Display the output.

PROGRAM  :-

from Crypto.Cipher import AES

from Crypto.Util.Padding import pad, unpad

key = b'thisismysecretky'

cipher = AES.new(key, AES.MODE_ECB)

plaintext = input("Enter message: ")

padded_text = pad(plaintext.encode(), AES.block_size)

encrypted_text = cipher.encrypt(padded_text)

print("Encrypted Text:", encrypted_text)

decrypted_text = unpad(cipher.decrypt(encrypted_text), AES.block_size)

print("Decrypted Text:", decrypted_text.decode())

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 05

18001

OUTPUT :-

CONCLUSION :-

In symmetric key cryptography, the same key is used for both encryption and decryption. DES and AES are two important symmetric algorithms, with AES being more secure and faster due to its larger key size and modern design.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 05

18001

PRACTICAL NO 06

STUDY OF ASYMETRIC KEY (DH & RSA)

AIM :- To Study and Implement Asymmetric Key Cryptography using DH & RSA Algorithms in Python.

ALORITHM OF DH  :-

1. Select a large prime number P and a primitive root G.

2. Each user selects a private key (random number).

Let user A choose private key a.

Let user B choose private key b.

3. Compute the public key:

A = G^a mod P

B = G^b mod P

4. Exchange public keys between both users.

5. Compute the shared secret key:

Key₁ = B^a mod P

Key₂ = A^b mod P

6. Both keys will be identical.

PROGRAM :-

P = 23  # prime number

G = 9   # primitive root

a = 4  # Alice's private key

b = 3  # Bob's private key

A = (G ** a) % P

B = (G ** b) % P

key1 = (B ** a) % P  # Alice's key

key2 = (A ** b) % P  # Bob's key

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 06

`                              18001

print("Publicly Shared Values:")

print("P:", P, "G:", G)

print("Public Key of Alice (A):", A)

print("Public Key of Bob (B):", B)

print("\nSecret Keys:")

print("Alice's Secret Key:", key1)

print("Bob's Secret Key:", key2)

OUTPUT :-

ALORITHM OF RSA  :-

1. Choose two prime numbers p and q.

2. Compute n = p × q.

3. Compute Euler’s totient function

φ(n) = (p − 1) × (q − 1).

4. Choose public key e such that

1 < e < φ(n) and gcd(e, φ(n)) = 1.

5. Compute private key d such that

(d × e) mod φ(n) = 1.

6. For encryption:

C = (M^e) mod n.

7. For decryption:

M = (C^d) mod n.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 06

18001

PROGRAM  :-

def gcd(a, b):

while b != 0:

a, b = b, a % b

return a

p = 7

q = 17

n = p * q

phi = (p - 1) * (q - 1)

e = 5

while gcd(e, phi) != 1:

e += 1

d = pow(e, -1, phi)

print("Public Key (e, n):", (e, n))

print("Private Key (d, n):", (d, n))

msg = int(input("Enter message (as number): "))

cipher = pow(msg, e, n)

print("Encrypted Message:", cipher)

decrypted = pow(cipher, d, n)

print("Decrypted Message:", decrypted)

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 06

18001

OUTPUT :-

CONCLUSION :-

Asymmetric cryptography uses two different keys for secure communication. In this practical DH was used to generate secret keys, and RSA was used for message encryption and  decryption, demonstrating secure data exchange.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 06

18001

PRACTICAL NO 07

STUDY OF MD5 ALGORITHM

AIM :- To study and implement the MD5 (Message Digest 5) hashing algorithm using Python.

ALGORITHM OF MD5 :-

1. Import the hash-lib library.

2. Accept a message from the user.

3. Encode the message to bytes.

4. Use hash-lib.md5() to compute the hash.

5. Display the hexadecimal digest.

PROGRAM :-

import hashlib

text = input("Enter text to hash: ")

md5_hash = hashlib.md5(text.encode())

print("MD5 Hash Value:", md5_hash.hexdigest())

OUTPUT :-

CONCLUSION :-

MD5 is a simple and efficient hashing technique that converts input data into a fixed-length digest.

It ensures data integrity by verifying whether data has been altered during transmission or storage.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 07

18001

PRACTICAL NO 08

STUDY OF HASH FUNCTION

AIM :- To study and implement the RSHash (Robert Sedgwicks Hash) function using Python.

PROGRAM :-

def RSHash(string):

a = 63689

b = 378551

hash_val = 0

for ch in string:

hash_val = hash_val * a + ord(ch)

a = a * b

return hash_val % (2**32)  # limit hash to 32 bits

text = input("Enter any text: ")

print("RSHash Value:", RSHash(text))

OUTPUT :-

CONCLUSION :-

RSHash is a simple and efficient hash function that generates a fixed-size numerical value for any input string.It is widely used for data lookup, hashing, and indexing, though it is not suitable for secure cryptographic applications.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 08

18001

PRACTICAL NO 09

TO CREATE, EXPORT AND VALIDATE A DIGITAL CERTIFICATE.

AIM :-  To write a code to Create, Export and Validate a digital Certificate.

- ALGORITHM :-       1. Download and Install opensol.

- 2.  Add its bin path in environmental variable .

- 3.  Open command prompt.

PROGRAM :-   C:\User\Payal\Desktop>openssl req -x509 -days 365 -newkey rsa:2048 -keyout private-key.pem -out certificate.pem

Enter PEM Pass: 123456

​​Country Name (2 letter code) []: IN

​State or Province Name (full name) []: Maharashtra

​Locality Name []: Mumbai

​Organization Name []: SIWS

​Organizational Unit Name []: IT

​Common Name []: TMD

​Email Address []: amalepayal044@gmail.com

C:\User\Payal\Desktop> openssl pkcs12 -export -in certificate.pem -inkey private-key.pem -out TMD.pfx

Enter Export Password: 123456

​Verifying - Enter Export Password: 123456

C:\User\Payal\Desktop> openssl pkcs12 -in TMD.pfx -clcerts -nokeys -out public-key.pem

Enter Import Password: 123456

Import the certificate

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 09

18001

OUTPUT :-

CONCLUSION :-    The above creation is done.

PAYAL AMALE                                                 S.I.W.S. College                                                    Major practical 09